Reneal 2026A Box and Project Notes

Version 1

IMPORTANT NOTES

- Ten new schools:

  - Kilimanjaro Region (Rombo DC): Tarakea, Mkuu, Mashati, Horombo

  - Ruvuma Region (Mbinga DC): Kigonsera

  - Ruvuma Region (Songea DC): Maposeni, Jenister Mhagama

  - Pwani Region (Dar es Salaam MC): Kibaha

  - Dodoma Region (Chamwino): Manchali Girls

  - Katavi Region (Mpimbwe): Mizengo Pinda

- There is one big difference in this shipment, compared to other shipments.  Four schools are receiving some Dell Thin Client Monitors with keyboard and mouse (instead of laptops).  These Thin Client Monitors look like a monitor but have a CPU, RAM, and storage inside.  They will legacy network boot perfectly to the Reneal server.  Just connect the Thin Client Monitor to the network switch through their RJ45 connector.  There is more information below on these units.  The 47 Thin Client Monitors are being deployed to the four new Kilimanjaro schools (Tarakea:12, Mkuu:12, Mashati:12, Horombo:11).  Laptops will be added so that each school has a total of 20 units.

- Another difference in this shipment is that two schools (Tarakea and Mkuu) will receive microdesktop servers instead of regular servers.

- I forgot to put the donation letters inside the System Admin manuals.  They donation letters are in Box U, inside a big envelope, with the network switches.  Please take the donation letters to each school to be signed.  Very sorry for that.

- There are more laptops this time with UEFI boot and other laptops with more modern drivers that would not boot to the Reneal server with 2025.A.UEFI software.  This means that the servers in this shipment have a new version of the Reneal server software, 2026.A.UEFI.  This software has replaced the former client image (4.4.0-210) with the latest version for 16.04 (4.15.0-142).  This should still be backward compatible for all laptops but work for newer laptop models.  Please note that all of the 47 spare laptops will boot to any version of the Reneal server software.

- There is also a lot of spare equipment in this shipment: 47 Dell laptops/adapters, a network switch, and two power supplies (Boxes Q, R, S, and part of V).  This goes to the Warehouse.

- Note that the following laptops must have batteries inside to operate: AS-2154, AS-2234, AS-2243.  Do not remove the batteries from these units!   Two other laptops had stripped screws and we could not get the laptop open to remove the battery (AS-2190, AS-2194) – please do not try to remove those batteries.

- All of these servers have Reneal Wireless.  Please watch the video (https://drive.google.com/file/d/1NTW8NvalmSxi2fIHIZJ83J-87Zmv8r4h/view?usp=drive_link) about how to set up these servers and use the Reneal wireless, and share this with the SUs.  For the desktop servers with the Reneal wireless, it is difficult to unplug the Reneal wireless router from the server LAN connection.  The small plastic “latch” on the RJ45 is hard to reach.  Please tell the SU to not pull it out, that will break the latch. They must use a plastic card like an ATM card or a piece of cardboard to reach in and push down the latch, so the RJ45 can be removed safely.  Please watch the video showing how to connect and disconnect.  Also note that to use the Reneal Wireless system with a cell phone, it may be necessary to turn off mobile data on the cell phone.

- The two microdesktop servers in this shipment are configured for the Reneal Wireless (they are not configured for internet).

- Many of the newer laptops have LAN ports like this.  Please tell the SUs that they must be very careful when inserting and removing the RJ45 connectors from these.  Sometimes the RJ45 is hard to insert and remove.  The SU must press the clip completely and gently “wiggle” the connector to insert and remove it.

- For any server with 16.04+webdev+wireless systems, the server MUST be plugged into the network switch for the virtual machines to work.  If the server is not plugged into the network switch, Kolibri will not start and the webdev system will not work.

General Notes

- Each school receives a mouse and keyboard for the server.  Each Dell Thin Client Monitor also has a mouse and a keyboard.

- Please collect any equipment donated by Apto that has failed and save it at the Reneal office.  We must return any dead items to Apto Solutions for recycling if we cannot fix them.

- In Boxes U and V, there are Team supplies (RJ45 connectors and cable ties) for installation.  There are extra RJ45 connectors for all Team members.  There is also some blue tape for the Reneal office.  There are some replacement keyboards for Dell 6000-series laptops.

Summary of Box Contents

- Here is the organization of the boxes:

  - Boxes A-D have either 8 or 9 laptops/adapters, plus a server keyboard and mouse and a wireless router for the 4 Kilimanjaro schools.  Boxes A and B also include a micro desktop Reneal server and the cables, plus the sys admin manual.

  - Boxes E-P are for the remaining 6 new schools.  They include 20 laptops/adapters, and a keyboard, a mouse, and a wireless router.  There are two boxes per new school (total of 12 boxes).

  - Boxes Q-S have the 47 spare Dell laptops/adapters.  Please take these boxes to the Warehouse.  These spare laptops can boot to any Reneal server.

  - THERE IS NO BOX T.

  - Box U has the 10 network switches for the new schools.  There is an envelope with two copies of the donation letters for each of the 10 new schools.  Some of the Team supplies are also in Box U.

  - Box V also includes the stands for the monitors for the 10 new schools, plus 2 monitor bases.  Box V also has a replacement network switch and 2 replacement power supplies.  These should go to the Warehouse.

  - There are 3 boxes (W, X, Y) that each have 12 monitor stands for the Dell Thin Client Monitors.  Box Z has 11 monitor stands for the Dell Thin Client Monitors.

  - Boxes AA, BB, CC, DD have the following:

    - Servers for the 8 remaining new schools (two per box).

    - The power cable for the server and the sys admin manual are inside each server box.

  - The 47 Dell Thin Client Monitors are going to the four new Kilimanjaro schools.  The boxes are packed as follows.  There are 7 boxes that each have 6 monitors, 6 keyboards, 6 mice, and 6 adapters (EE, FF, GG, HH, II, JJ, KK).  Box LL has 5 monitors/keyboards/mice/adapters.

  - Box MM and Box NN have the monitor screens, video cables, and power cables for the 10 new schools.  These boxes also include the monitor bases for 8 of the new schools.  The monitor parts are labeled with numbers 1 to 10 (one per new school).

- All of the monitors use VGA cables.

- The following boxes have all of the equipment for the 10 new schools:

  - Tarakea Secondary School: A, EE, FF, W, part of U, part of V, part of MM

  - Mkuu Secondary School: B, GG, HH, X, part of U, part of V, part of MM

  - Mashati Secondary School: C, II, JJ, Y, part of U, part of V, part of AA, part of MM

  - Horombo Secondary School: D, KK, LL, Z, part of U, part of V, part of AA, part of MM

  - Kigonsera Secondary School: E, F, part of U, part of V, part of BB, part of MM

  - Maposeni Secondary School: G, H, part of U, part of V, part of BB, part of NN

  - Jenister Mhagama Secondary School: I, J, part of U, part of V, part of CC, part of NN

  - Kibaha Secondary School: K, L, part of U, part of V, part of CC, part of NN

  - Manchali Girls Secondary School: M, N, part of U, part of V, part of DD, part of NN

  - Mizengo Pinda Secondary School: O, P, part of U, part of V, part of DD, part of NN

- Team supplies are in Box U (RJ45 connectors for the new installations, zip ties) and Box V (extra RJ45 connectors for the Team, Dell 6000-series replacement keyboards, tape for the office).

- Boxes Q, R, S and part of V are spares and go to the Warehouse.

NOTE: There is no Box T.

Dell Thin Client Monitors

The BIOS for the Dell Thin Client Monitors is set for legacy boot. To access BIOS, you type the Delete key when you see the Dell logo.  Then you must enter the BIOS password (Fireport).  The storage device for each of Thin Client Monitors has been wiped, so the system should automatically network boot, even if the CMOS battery dies.

Please note – disassembly of the Dell Thin Client Monitor is difficult.  While the CMOS battery can be changed, I do not recommend trying it.  I made videos of the disassembly process.  Please let me know if you would like to see what it looks like inside.

To use the Dell Thin Client Monitors units, plug in an Ethernet cable from the network switch to the Dell Thin Client Monitor RJ45 port.  Plug in the keyboard and mouse.  Plug in the power adapter.  Turn it on.  You can adjust the audio volume and monitor brightness using the buttons on the front of the monitor.  Some of them may have their audio volume set very low, so please check this during installation.

Reneal Wireless System

Wifi signal: LAB_2G or LAB_5G (Password 12345678)

Navigate to 10.10.10.1

Teachers may need to turn off mobile data to use the Reneal Wireless on their phones or tablets.

The Reneal Wireless router (use an orange plug) connects to the network interface card in the server that has one port.  Tell the SUs to not reconfigure the Reneal wireless router.  If they do, the Reneal Wireless will not work.  The Reneal wireless router does not provide internet, only the Reneal educational material on the server.  If the school has internet, it should be plugged into the LAN connector on the motherboard, as usual.

Arusha Equipment

| Item | Purchased | Destination |
| --- | --- | --- |
| Uninterruptible power supply (UPS) | 10 | All schools |
| Voltage stabilizer, 500VA | 10 | All schools |
| Power universal extensions (5 outlets) | 50 | 5 each for all schools |
| UTP cable | 5 boxes | One box for two schools |

I will also try to purchase replacement UPS and voltage stabilizer units for schools that need them.  I will update you when I know how many are needed.

Installation Notes

Here is a list of what you will need at TARAKEA and MKUU:

Micro desktop server, power adapter for server, RJ45-USB dongle for server, UPS power cable adapter for server, video adapter for server, sysadmin manual and 2 donation letters, keyboard and mouse for server, monitor screen, monitor stand (2 pieces), power and video cables for monitor, network switch and power cord, 8 laptops and laptop adapters, 12 Dell Thin Client Monitors each with adapters/mice/keyboards, a wireless router with power adapter and Ethernet cable, UPS, 2 special UPS cords (for monitor, and network switch), voltage stabilizer, 5 extensions, Cat6 cable, RJ45 connectors, zip ties, blue tape, pens, cable clips, screws for mounting the switch, hammer and drill, your personal supplies and tools, power meter, and Reneal T-shirts.

Here is a list of what you will need at MASHATI:

Server, power cord for server, sysadmin manual and 2 donation letters, keyboard and mouse for server, monitor screen, monitor stand (2 pieces), power and video cables for monitor, network switch and power cord, 8 laptops and laptop adapters, 12 Dell Thin Client Monitors each with adapters/mice/keyboards, a wireless router with power adapter and Ethernet cable, UPS, 3 special UPS cords (for server, monitor, and network switch), voltage stabilizer, 5 extensions, Cat6 cable, RJ45 connectors, zip ties, blue tape, pens, cable clips, screws for mounting the switch, hammer and drill, your personal supplies and tools, power meter, and Reneal T-shirts.

Here is a list of what you will need at HOROMBO:

Server, power cord for server, sysadmin manual and 2 donation letters, keyboard and mouse for server, monitor screen, monitor stand (2 pieces), power and video cables for monitor, network switch and power cord, 9 laptops and laptop adapters, 11 Dell Thin Client Monitors each with adapters/mice/keyboards, a wireless router with power adapter and Ethernet cable, UPS, 3 special UPS cords (for server, monitor, and network switch), voltage stabilizer, 5 extensions, Cat6 cable, RJ45 connectors, zip ties, blue tape, pens, cable clips, screws for mounting the switch, hammer and drill, your personal supplies and tools, power meter, and Reneal T-shirts.

Here is a list of what you will need at THE REMAINING SIX SCHOOLS:

Server, power cord for server, sysadmin manual and 2 donation letters, keyboard and mouse for server, monitor screen, monitor stand (2 pieces), power and video cables for monitor, network switch and power cord, 20 laptops and laptop adapters, a wireless router with power adapter and Ethernet cable, UPS, 3 special UPS cords (for server, monitor, and network switch), voltage stabilizer, 5 extensions, Cat6 cable, RJ45 connectors, zip ties, blue tape, pens, cable clips, screws for mounting the switch, hammer and drill, your personal supplies and tools, power meter, and Reneal T-shirts.

Other important notes:

- Before you plug anything in, check the computer room voltage with the power meter to make sure it is around 220 volts and stable.

- PLEASE NOTE:  UPS should be charged for 12 hours before it is used the first time.

- Three items – the server, server monitor, and network switch – must be plugged into the UPS.

- Please use the installation checklist to make sure all steps are completed and you gather all the needed information about the new school plus the signed donation letters.  The donation checklist, GPS coordinates, and signed donation letter should be sent to Erick as soon as possible to put on the Reneal website.

- If the server does not start properly, take off the cover and check the power connectors to the motherboard, check any SATA cables, make sure the RAM is inserted fully, and check the network cards to make sure they are inserted completely.  The main storage device is now an Nvme stick attached to the motherboard, so they should not need to be checked since they are firmly attached.

- I have the specifications for all of the laptops in a spreadsheet.  Please leave the blue tape with the number on the laptop, for identification.  “AS” with a number means the laptop/adapter was donated by Apto Solutions.

- For a regular Reneal server with webdev+wireless:  The internet, if available, plugs into the motherboard RJ45 connector.  The Reneal Wireless router plugs into the network interface card with one port.  The network switch has two connections to the server, which plug into the network interface card with two ports.

- For a microdesktop Reneal server with webdev+wireless: The network switch should be connected to the LAN connection in the microdesktop.  The Reneal Wireless router plugs into the LAN-USB dongle.  The microdesktop servers are configured for Reneal Wireless, not for internet.

Laptop Notes

All of the laptops in this shipment are Dell.

There are two laptop models (3400: 1 and 3500:3) at Mizengo Pinda that use a smaller connector for the laptop power.  There are short cables to convert the big connector to the small connector for each of those laptops.

Some of the laptops are very large.  I would suggest that you try to attach a big adapter to them.  Otherwise, all of the Dell adapters should work with any Dell laptop, so the adapters are not labeled with a number.  For some Dell laptops, you may see a message about the power adapter not providing enough power to charge the battery, or that it can’t identify the power adapter.  Please just hit F3, which will disable future warnings.  Then hit F1 to continue the boot.

Software Notes

This is a new software release!  It is 2026.A.UEFI.  This new version was necessary because some of the laptops being deployed in 2026 Phase 1 will not work with the 2025.A software release.  The client image in 2026.A.UEFI was updated to the 4.15.0-142 kernel (compared to 4.4.0-210 in 2025.A and 2025.A.UEFI).  This new kernel provides updated drivers needed for new laptops.  [Please note that an additional update to a version of kernel 5.15.0 will probably be necessary for 2026 Phase 2, but it is still being tested.]

Mizengo Pinda has 19 laptops that are only UEFI boot (not legacy boot).  You will notice they look a little different when they boot, but they will end at the blue Welcome screen.

All 10 of the new servers have all of the new National Exam papers.

Box Contents

| Box | Item | Quantity | Destination |
| --- | --- | --- | --- |
| A | Laptop | 8 | Tarakea |
| A | Laptop adapter | 8 | Tarakea |
| A | Mouse | 1 | Tarakea |
| A | Keyboard | 1 | Tarakea |
| A | Wireless router | 1 | Tarakea |
| A | Microdesktop/cables | 1 | Tarakea |
| A | Sysadmin Manual | 1 | Tarakea |
| Box | Item | Quantity | Destination |
| B | Laptop | 8 | Mkuu |
| B | Laptop adapter | 8 | Mkuu |
| B | Mouse | 1 | Mkuu |
| B | Keyboard | 1 | Mkuu |
| B | Wireless router | 1 | Mkuu |
| B | Microdesktop/cables | 1 | Mkuu |
| B | Sysadmin Manual | 1 | Mkuu |
| Box | Item | Quantity | Destination |
| C | Laptop | 8 | Mashati |
| C | Laptop adapter | 8 | Mashati |
| C | Mouse | 1 | Mashati |
| C | Keyboard | 1 | Mashati |
| C | Wireless router | 1 | Mashati |
| Box | Item | Quantity | Destination |
| D | Laptop | 9 | Horombo |
| D | Laptop adapter | 9 | Horombo |
| D | Mouse | 1 | Horombo |
| D | Keyboard | 1 | Horombo |
| D | Wireless router | 1 | Horombo |
| Box | Item | Quantity | Destination |
| E | Laptop | 9 | Kigonsera |
| E | Laptop adapter | 20 | Kigonsera |
| E | Mouse | 1 | Kigonsera |
| E | Keyboard | 1 | Kigonsera |
| E | Wireless router | 1 | Kigonsera |
| Box | Item | Quantity | Destination |
| F | Laptop | 11 | Kigonsera |
| Box | Item | Quantity | Destination |
| G | Laptop | 9 | Maposeni |
| G | Laptop adapter | 20 | Maposeni |
| G | Mouse | 1 | Maposeni |
| G | Keyboard | 1 | Maposeni |
| G | Wireless router | 1 | Maposeni |
| Box | Item | Quantity | Destination |
| H | Laptop | 11 | Maposeni |
| Box | Item | Quantity | Destination |
| I | Laptop | 9 | Jenister Mhagama |
| I | Laptop adapter | 20 | Jenister Mhagama |
| I | Mouse | 1 | Jenister Mhagama |
| I | Keyboard | 1 | Jenister Mhagama |
| I | Wireless router | 1 | Jenister Mhagama |
| Box | Item | Quantity | Destination |
| J | Laptop | 11 | Jenister Mhagama |
| Box | Item | Quantity | Destination |
| K | Laptop | 9 | Kibaha |
| K | Laptop adapter | 20 | Kibaha |
| K | Mouse | 1 | Kibaha |
| K | Keyboard | 1 | Kibaha |
| K | Wireless router | 1 | Kibaha |
| Box | Item | Quantity | Destination |
| L | Laptop | 11 | Kibaha |
| Box | Item | Quantity | Destination |
| M | Laptop | 9 | Manchali Girls |
| M | Laptop adapter | 20 | Manchali Girls |
| M | Mouse | 1 | Manchali Girls |
| M | Keyboard | 1 | Manchali Girls |
| M | Wireless router | 1 | Manchali Girls |
| Box | Item | Quantity | Destination |
| N | Laptop | 11 | Manchali Girls |
| Box | Item | Quantity | Destination |
| O | Laptop | 10 | Mizengo Pinda |
| O | Laptop adapter | 20 | Mizengo Pinda |
| O | Mouse | 1 | Mizengo Pinda |
| O | Keyboard | 1 | Mizengo Pinda |
| O | Wireless router | 1 | Mizengo Pinda |
| Box | Item | Quantity | Destination |
| P | Laptop | 10 | Mizengo Pinda |
| Box | Item | Quantity |  |
| Q | Laptop | 16 | Warehouse |
| Q | Laptop adapter | 16 | Warehouse |
| Box | Item | Quantity |  |
| R | Laptop | 16 | Warehouse |
| R | Laptop adapter | 16 | Warehouse |
| Box | Item | Quantity |  |
| S | Laptop | 15 | Warehouse |
| S | Laptop adapter | 15 | Warehouse |
| Box | Item | Quantity | Destination |
| U | Network Switches | 10 | All Schools |
| U | Team Supplies |  | Team |
| U | Donation letters |  |  |
| Box | Item | Quantity | Destination |
| V | Network Switch | 1 | Warehouse |
| V | Monitor stand | 10 | All schools |
| V | Team supplies |  | Team |
| V | Spare power supply | 2 | Warehouse |
| Box | Item | Quantity |  |
| W | Client monitor stand | 6 | Tarakea |
| Box | Item | Quantity |  |
| X | Client monitor stand | 6 | Mkuu |
| Box | Item | Quantity |  |
| Y | Client monitor stand | 6 | Mashati |
| Box | Item | Quantity |  |
| Z | Client monitor stand | 5 | Horombo |
| Box | Item | Quantity | Destination |
| AA | Desktop computer | 1 | Mashati |
| AA | Desktop computer | 1 | Horombo |
| AA | Sysadmin Manual | 1 | Mashati |
| AA | Sysadmin Manual | 1 | Horombo |
| Box | Item | Quantity | Destination |
| BB | Desktop computer | 1 | Kigonsera |
| BB | Desktop computer | 1 | Maposeni |
| BB | Sysadmin Manual | 1 | Kigonsera |
| BB | Sysadmin Manual | 1 | Maposeni |
| Box | Item | Quantity | Destination |
| CC | Desktop computer | 1 | Jenister Mhagama |
| CC | Desktop computer | 1 | Kibaha |
| CC | Sysadmin Manual | 1 | Jenister Mhagama |
| CC | Sysadmin Manual | 1 | Kibaha |
| Box | Item | Quantity | Destination |
| DD | Desktop computer | 1 | Manchali Girls |
| DD | Desktop computer | 1 | Mizengo Pinda |
| DD | Sysadmin Manual | 1 | Manchali Girls |
| DD | Sysadmin Manual | 1 | Mizengo Pinda |
| Box | Item | Quantity |  |
| EE | Client monitor | 6 | Tarakea |
| EE | Adapter | 6 | Tarakea |
| EE | Keyboard | 6 | Tarakea |
| EE | Mouse | 6 | Tarakea |
| Box | Item | Quantity |  |
| FF | Client monitor | 6 | Tarakea |
| FF | Adapter | 6 | Tarakea |
| FF | Keyboard | 6 | Tarakea |
| FF | Mouse | 6 | Tarakea |
| Box | Item | Quantity |  |
| GG | Client monitor | 6 | Mkuu |
| GG | Adapter | 6 | Mkuu |
| GG | Keyboard | 6 | Mkuu |
| GG | Mouse | 6 | Mkuu |
| Box | Item | Quantity |  |
| HH | Client monitor | 6 | Mkuu |
| HH | Adapter | 6 | Mkuu |
| HH | Keyboard | 6 | Mkuu |
| HH | Mouse | 6 | Mkuu |
| Box | Item | Quantity |  |
| II | Client monitor | 6 | Mashati |
| II | Adapter | 6 | Mashati |
| II | Keyboard | 6 | Mashati |
| II | Mouse | 6 | Mashati |
| Box | Item | Quantity |  |
| JJ | Client monitor | 6 | Mashati |
| JJ | Adapter | 6 | Mashati |
| JJ | Keyboard | 6 | Mashati |
| JJ | Mouse | 6 | Mashati |
| Box | Item | Quantity |  |
| KK | Client monitor | 6 | Horombo |
| KK | Adapter | 6 | Horombo |
| KK | Keyboard | 6 | Horombo |
| KK | Mouse | 6 | Horombo |
| Box | Item | Quantity |  |
| LL | Client monitor | 5 | Horombo |
| LL | Adapter | 5 | Horombo |
| LL | Keyboard | 5 | Horombo |
| LL | Mouse | 5 | Horombo |
| Box | Item | Quantity | Destination |
| MM | Monitor screen | 5 | Tarakea, Mkuu, Mashati, Horombo, Kigonsera |
| MM | Monitor base | 3 |  |
| MM | VGA/power cable | 5 | Tarakea, Mkuu, Mashati, Horombo, Kigonsera |
| Box | Item | Quantity | Destination |
| NN | Monitor screen | 5 | Maposeni, Jenister Mhagama, Kibaha, Manchali Girls, Mizengo Pinda |
| NN | Monitor base | 5 |  |
| NN | VGA/power cable | 5 | Maposeni, Jenister Mhagama, Kibaha, Manchali Girls, Mizengo Pinda |